Gram :  CENBOSEC, Delhi – 92 	                                                                               Phone: 22509256-59
E-Mail:  cbsedli@nda.vsnl.net.in 	                                                                                                   Fax:    22515826	
website: www.cbse.nic.in

CENTRAL BOARD OF SECONDARY EDUCATION 
(An Autonomous Organisation under the Union Ministry of Human Resource Development Govt. of India)

“SHIKSHA KENDRA”, 2, COMMUNITY CENTRE, PREET VIHAR, DELHI – 110092

No. F-119/09/					 			Date: 21.12.2010

……………………………..

……………………………..

……………………………..

……………………………..

Sub:- Limited Tender for Reinforcement of roof slabs in LIG/MIG flats at Brij 		 Vihar, Ghaziabad.

Sir,

. The Board intends to invite Limited Tenders for the above cited work from reputed agencies which have at least executed two similar works. Therefore your firm is advised to quote the rates on the prescribed format alongwith an EMD of Rs. 10,000/- payable in f/o the Secretary CBSE through DD/BD. The price-bid must be placed in a separate sealed envelope. The sealed tender complete in all respect must be submitted latest by 04.01.2011 upto 2.00 p.m. and dropped in the Tender-Box placed at Ground Floor near Reception Counter, CBSE HQ Building, Preet Vihar, Delhi-110092. Your firm is also advised to visit the said flats before filling up the tender so that exact quantum of work is assessed. Since the work is specialized, the agency shall be required to submit an affidavit ensuring five years guarantee for the executed work.

Incomplete & conditional tenders shall be summarily rejected.

Yours faithfully

(S.K. SHARMA)

SR. ENGINEER (ESTATE)

SCHUDULE OF RATE

(OPTION-I)

| S.No. | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling old plaster or skirting raking out joints and cleaning the surface for plaster i/c disposal of rubbish to the dumping ground with in 50 metres lead. | Sqm | 165 |  |  |
| 2. | Chipping of unsound/ weak concrete material from slabs, beams, columns manually or with power driven percussion type or pneumatic chisel of standard make as directed by Engineer-in-charge i/c tapering of all edges, making square shoulders of cavities etc. complete and disposal of debris upto 50mt and all lifts. (a) Upto required depth. | Sqm | 165 |  |  |
| 3. | Cleaning of rust from exposed reinforcement bars to give it a rust free finished steel surface, by using alkaline chemical rust remover of ROFFS, PIDILITE, FIX-IT or equivalent make with paint brush and removing loose particles after 24 hours of its application with wire brush and thoroughly washing with water and allowing it to dry. (Surface area of slab, beams, columns shall be measured for payment). | Sqm | 165 |  |  |
| 4. | Providing, mixing and applying over reinforcement bars zinc anti rust agent of FOSROC, CICO, PIDILITE or equivalent make as per manufactures specification and direction of Engineer-in-charge. (Surface area of slab, beam, column shall be measured for payment) | Sqm | 140 |  |  |
| 5. | Drilling suitable holes in reinforced or plain cement concrete with high speed drill depth upto 100-150mm in RCC beam, lintels, column and slabs to introduce steel bars for sunshades/ balconies i/c fixing the steel bars in position using epoxy but excluding the cost of reinforcement. (a) Any dia meter of steel bar. | Each | 200 |  |  |
| 6. | Providing Polymer modified cement plaster in layers, each layer not exceeding 10mm in thickness, complete as per direction of Engineer-in-charge over the repaired surface in cement mortar 1:3 (1 cement:3 fine sand) | Sqm | 165 |  |  |
| 7. | Wet curing of shotcreted or plastered surface of RCC work as per specifications by keeping it continuously wet for a minimum period of seven days. Using pre-tested and approved water based concrete curing compound. | Sqm | 165 |  |  |
| 8. | Disposal of malba/bldg. rubbish by mechanical transport i/c loading, unloading and stacking in approved municipal dumping ground for all lead & lift etc. complete. | Cum | 15 |  |  |
| 9. | Reinforcement of RCC work i/c straightening cutting bending, binding, placing in position. (a) T.M.T. bars | Kg. | 1800 |  |  |
| 10. | Providing and fixing hard drawn steel wire mesh of 75x75x4mm of weight not less than 2.70kg/sqm. of more in item of shotcreting, with necessary nails, binding wire etc. complete and as per direction of Engineer-in-charge. | Sqm | 100 |  |  |
| 11. | Distempering with oil bound washable distemper of approved brand and manufacture to give an even shade-New work (Two or more coats) over and including coat with cement primer. | Sqm | 165 |  |  |
| 12. | Supplying and applying nafulfill BB-2 of MC or equivalent @ 0-214kg/m2 mixed with cement slurry (cement used 2.2kg/sqm in slurry or as per manufacture’s recommendation) the chipped off surface of RCC roofs, ceiling, beams, columns, chajjas walls etc. to make perfect bonding between the old RCC and new shotcreting as per direction of Engineer-in-charge. (1) Premeasurement of area where BB-2/cememt slurry is to be applied shall be measured as per direction of Engineer-in-charge. | Sqm | 165 |  |  |

Accepted by me:

Gross Value:

Signature

Prop./Manager

Tel. No.

Mob. No.

SCHEDULE OF RATE

(OPTION-II)

| Sl. No. | Description of Item | Qty | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling of loose / weak cement concrete, cutting the rusted reinforcement and replacement in the new steel bars duly welded. The old reinforcement shall be exposed & chemically treated with 2 coats of bipolar corrosion inhibitor (EPCO KP. 100) & FEOVERT. Subsequently, the ceiling shall be repaired through an admixture of stone aggregate maximum (10mm), cement, coarse sand and ASTM C-1059-86 Type II (Epoxy. latex additive) in 2/3 layers as per site requirement. Note: - Reinforcement shall be paid separately. | 165 (approx) | Sq-m |  |  |
| 2. | Reinforcement for R.C.C. works including straightening, cutting, bending, placing, binding etc. all complete. (mild steel and medium tensile steel bars.) | 2000 (approx) | k.g |  |  |

Gross Value:

Signature

Prop./Manager

Tel. No.

Mob. No.

Terms & Conditions

- The work shall be executed as per the approved specifications.

- The work is time bound, hence time is the essence of the contract.

- The work is to be completed within 3 months from 7th day of award of work.

- In one premises, the work must be completed within 12 days.

- No T & P shall be provided by the Board.

- Scaffolding shall be arranged by the Contractor.

- The work shall be executed under the strict supervision of the Board’s J.E.

- The measurements shall be recorded jointly.

- The agreement certifying five years life shall be given on Rs.10/- stamp paper duly attested by the Notary Public.

- Potable Water shall only be used. Ground water shall not be used at all even for the curing purposes.

- In case of delay a penalty @ 1% per week shall be levied.

- In case of poor workmanship, the Board shall have the right to rescind the contract and get the work executed through any other agency at the risk & cost of the defaulting contractor.

- The contractor shall be required to provide adequate safety for its workers and the Board shall not be accountable to any kind of injury/ accident at site.

- A sum of Rs.30000/- shall be retained as security for a period of 6 months as defect liability. In case no defect is observed the same shall be released.

Accepted by me

Signature

Prop./Manager

Tel. No.

Mob. No.