Gram :  CENBOSEC, Delhi – 92 	                                                                               Phone: 22509256-59
E-Mail:  cbsedli@nda.vsnl.net.in 	                                                                                                   Fax:    22515826	
website: www.cbse.nic.in

CENTRAL BOARD OF SECONDARY EDUCATION 
(An Autonomous Organisation under the Union Ministry of Human Resource Development Govt. of India)

“SHIKSHA KENDRA”, 2, COMMUNITY CENTRE, PREET VIHAR, DELHI – 110092

SE/Estates/2010/F-1/251/                                                                                  11/06/2010

_____________________________

_____________________________

_____________________________

_____________________________

Sub: -   Limited Tender for P & F of PVC doors and Ceramic Floor Tile in LIG flats                                                              	  at Brij Vihar, Ghaziabad.

Sir,

Sealed tenders are invited from reputed agencies for providing and fixing of PVC doors, ceramic antiskid floor tiles through specialized agencies which must have executed two (2) similar works earlier. The rates are to be quoted on the prescribed format. An EMD amounting to Rs. 10000/- payable in the form of DD/BD in f/o the Secretary CBSE  must be enclosed with the tender. The duly completed be submitted on or before 25.06.2010 up to 2.00 pm. Incomplete and conditional tenders shall be summarily rejected. The Secretary reserves the right to reject any or all the tenders without assigning any reasons thereof.

Yours faithfully

(S.K. SHARMA)

Senior Engineer (Estate)

| Sl. No. | Description of Item | Qty | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
|  | Providing and fixing 28 mm thick door shutter made of solid PVC foam profile with homogenous fine cellular structure having smooth outer integral skin having 71mm width & 28mm thick as styles and rails. Joints are made using solvent adhesive and GI 'C' sections (39mm x 19mm x 0.6mm thick) or M S pipe (40mm x 20mm ) stiffener frame insert & telescopic polymeric 'L' corners .The panel shall be filled with 3mm thick high - pressure compact laminate as per manufacturer's specifications and direction of Engineer-in-charge, cover moulding shall be provided for covering fixing screws and elegant look.(for W.C. and bathroom door shutter).The brand approved shall be “Rajshree” . | 73.10 | Sqm |  |  |
|  | Dismantling and buying-back the old wooden doors with M.S Angle/ Tee Section ( size 50x50x5mm) | 43 | Each |  |  |
|  | Providing and fixing factory made door frame (single rebate) made of solid PVC foam profile with homogenous fine cellular structure having smooth outer integral skin having 60mm width & 30mm thickness and shall be fixed to wall as per instructions of engineer-in-charge using 100x8 sheet metal CSK screws. The approved brand shall be “Rajshree”. | 207 | metre |  |  |
|  | Providing and laying Ceramic antiskid glazed floor tiles 300x300 mm (thickness7-8mm) of 1st quality conforming to IS : 15622 of approved make in colours such as White, Ivory, Grey, Fume Red Brown, laid on 20 mm thick Cement Mortar 1:4 (1 Cement : 4 Coarse sand) including pointing the joints with white cement and matching pigment etc., complete. | 95 | Sqm |  |  |
|  | Dismantling P.C.C flooring in W.C& Bath including stacking of serviceable material and disposal of unserviceable material within 50 metres lead. | 95 | Sqm |  |  |
|  | Disposal of malba from worksite to dumping site within 5 k.m lead. | 10 | Cu.m |  |  |

Gross Value:-

Signature Prop. /Manager

Terms & Conditions

- The work shall be executed as per the approved specifications.

- The work is time bound; hence time is the essence of the contract.

- the work is to be completed within 1 month from 7th day of award of work.

- No T & P shall be provided by the Board.

- The work shall be executed under the strict supervision of the Board’s J.E.

- The measurements shall be recorded jointly.

- In case of delay a penalty @ 1% per week shall be levied. The max. penalty shall be 5%.

- In case of poor workmanship, the Board shall have the right to rescind the contract and get the work executed through any other agency at the risk & cost of the defaulting contractor.

- A sum of Rs. 30000/- shall be retained as security for a period of 6 months as defect liability. In case no defect is observed the same shall be released.

Accepted by me

Signature

Prop. /Manager

Tel. No.

Mob. No.