AE/Estates/2008/                                                                                                                 17.12.2008

_____________________________

_____________________________

_____________________________

_____________________________

Sub: Anti -Termite Treatment in Publication Store at Shiksha Sadan Bldg.

17, Rouse Avenue, New Delhi

Sir,

The CBSE intends to get the Anti-Termite treatment work executed in its Publication Store at Shiksha Sadan Bldg., 17 Rouse Avenue, New Delhi measuring approx. 8070 sq. ft.  Specialized agencies executing similar works may apply on the enclosed prescribed format alongwith an EMD of Rs. 2000/- payable in favour of the Secretary CBSE either through BD/DD/Cash.  The quotation must be submitted in a sealed envelope superscribed “Anti-Termite Treatment” latest by 31.12.08 upto 2.00 p.m.  and dropped in the Tender Box placed near Reception Counter at Ground Floor, CBSE HQ. Building Preet Vihar Delhi- 110092. The quotation shall be opened on the same day at 2:30 pm .  Incomplete & conditional tenders shall be summarily rejected.

Yours faithfully,

(S. K. SHARMA )

ASSTT. ENGR. (ESTATE)

SCHEDULE OF RATE

| Sl. No. | Description of Item | Qty | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Supply chemical emulsion in sealed containers including delivery as specified. Chlorpyriphas / lindane emulsifiable concentration of 20 % | 20 | litre |  |  |
| 2 | Diluting and injecting chemical emulsion for POST CONSTRUCTIONAL antitermite treatment ( including the cost of chemical emulsion ) Treatment of soil under existing floors / walls using chemical emulsion @ one liter per hole 300mm apart including drilling 12mm diameter holes and plugging with cement mortar 1:2 ( 1cement : 2 coarse sand ) to match the existing flooring With Chlorphyriphas / lindane E.C 20% with 1% concentration. | 750 (approx ) | Sq-m |  |  |

TERMS & CONDITIONS

- The agency shall have to execute the work as per the C.P.W.D specifications as well as the directions of the Engineer Incharge.

- The agency shall have to given an undertaking that anti-termite work would be effective for 5 years.

- The work shall have to be completed within 20 days from the Date of start.

- In case of extra item, written permission need to be obtained.

- In case of defect liability the agency shall be bound to attend the complaint within 48 years.

- The measurements shall be done jointly by the agency as well as the J.E.

- In case of delay, penalty @ 1% per week shall be levied.

- In case of inferior work, the agency shall be bound to redo the work.  In this regard, the decision of the Engineer-In charge would be final.

- The agency shall depute a supervisor to coordinate timely execution in consultation with the Store Keeper/JE.

Accepted by me

Prop/Manager

Seal of the firm

Tel. No.:__________________

Mobile No.:________________