AE/Estates/2008/                                                                                                17.03.2008

_____________________________

_____________________________

_____________________________

_____________________________

Sub: Quotation for Wooden Cabinet in the CM’s Secretaiat

Sir,

The CBSE intends to get Wooden Cabinet work executed as per the schedule &  the attached drawing.  Interested agencies may apply on the prescribed format alongwith an EMD of Rs. 1000/- through BD/DD payable in favour of the Secretary CBSE.  Quotation complete in all respect,  be placed in an envelope duly sealed & dropped in Tender Box placed at Reception Hall, Ground Floor, HQ Building, Preet Vihar on or before 27.03.08 upto 2.00 p.m..  Incomplete & conditional quotations shall be summarily rejected.  The Secretary reserves the right to reject any or all the quotations without assigning any reason thereof.

Yours faithfully,

(S. K. SHARMA )

ASSTT. ENGR. (ESTATE)

Name of Work : Providing & Fixing of Wooden Cabinet in CM’s Sectt.

SCHDULE OF WORK

Sl. No.	   Description of Item				       Qty  Unit  Rate 	Amount

- Providing & fixing of wooden cabinet 			7.0 m2

comprising lockable cupboard with shelves,

remaining area with open selvels partitioned apportionately with archial supports, as well as the vertical support in monotomy with the existing wooden storage cabinets complying the enclosed drawing.  The constructional parameters are as under :

- The bottom shall have a frame of well seasoned hollock wood.

- The shelves shall be made with 18 mm thick flat pressed 3 layers or graded wood particle board medium density grade I & wood lipping complete.

- The cupboard shutters shall be made with 18 mm thick flat pressed 3 layers or graded wood particle board having veneers on one side.

- The wooden moulding/beading shall be of teak wood (2nd class) fixed on edges of frames with headless nails & glued properly.

- The cupboard shall have 75 mm ISI Marked butt hinges (6 nos.)

- Brass handles (2 Nos.) of size 150 mm (excellent quality)

- Painting of the Cup-board (interior) with		10 m2

Nerolac/Berger paint after applying

wood primer & proper dressing/sand

papering.

- French spirit polishing with two or more 		20 m2

coats on new work i/c a coat of wood filler.

- P & F brass cup-board lock 6 levers 65 mm		1 No.

(as per approved quality)  i/c key hole.

- P & F brass tower bolt 100 mm x 8 mm with		1 No.

brass screws.

Terms and Conditions

- The work has to be executed as per the drawings/directions of the  Engineer-In-Charge.

- The material used shall be got checked before installation.

- The workman ship must be excellent.

- The work shall have to be executed during weekly off days i/c late hours so that office working is not disturbed.

- The work has to be completed within 20 days of the issue of the work order.

- In case of delay, a penalty of 1% per week shall be levied.

- No running payment shall be made

- The wood work has to be matched with the existing wooden cabinets i/c the veneers.

- Quotations without EMD shall not be considered.

Accepted by me

Prop/Manager

Seal of the firm

Tel. No.:__________________

Mobile No.:________________